CORRECTIVE ACTION REPORT

Description of Non Confirmity(NC): During in-process inspection, a batch of machined components was found with dimensional deviation beyond tolerance limits, specifically in outer diameter (OD). The deviation resulted in poor fitment during assembly. Approximately 4% parts from the lot were identified as non-conforming, requiring segregation and rework. This affected the production schedule and delayed onward dispatch. Identified by: Muskan Accepted By: Hemant

Root Cause Analysis: The root cause was traced to improper tool offset settings on the CNC machine due to the operator not updating the offset after tool wear. Secondary contributing factors included insufficient frequency of tool wear monitoring and lack of a clear checklist for offset verification during shift change. The absence of a formal cross-check mechanism allowed the deviation to go unnoticed for multiple cycles.

Corrective Action: The CNC operator was instructed to immediately update tool offsets based on actual gauge measurements. The complete batch was 100% rechecked, and all non-conforming parts were segregated for rework. A supervisor performed cross-verification of offsets before restarting production. Tool inserts showing excessive wear were replaced, and machining parameters were stabilized before releasing the machine for production again. Target Completion Date: 18/04/2026

Preventive Action: A mandatory tool offset verification checklist has been introduced for all CNC machines at the start of each shift. The frequency of tool wear inspection has been increased from once per shift to twice per shift. Training has been provided to operators on offset control and dimensional accuracy requirements. A quality gate has been added after the CNC operation for random sampling to detect early abnormalities. Documentation control has been strengthened to ensure traceability of offset changes.

Verification for Effective Implementation: Post-implementation audits confirmed that dimensional deviations have reduced to less than 0.5%, and no further NCs were observed in subsequent production batches. Quality Inspector verified checklist compliance for three consecutive days and confirmed adherence. Monitoring records indicate improved stability in machining accuracy, confirming successful closure of the CAPA. Verified by: Hemant Date: 25/04/2026