Risk Assessment Method: Severity (S) × Likelihood (L) = Risk Rating (R)

Scale:

- Severity: 1 (Minor) – 5 (Fatal)

- Likelihood: 1 (Rare) – 5 (Frequent)

- Risk Level: Severity X Likelihood = R

- Significance Criteria:

  - R: - 1–5 = Low

  - R: - 6–10 = Medium

  - R: - 11–15 = High (Mandatory Further Actions)

  - R: - 16–25 or Severity 5 = Very High (Mandatory Further Actions)

1. RAW MATERIAL INWARD & MATERIAL INSPECTION, STORAGE

| Hazard | Risk | S | L | R | Control Measures | Further Action |
| --- | --- | --- | --- | --- | --- | --- |
| Manual handling | Back strain | 3 | 3 | 9 (M) | Trolley use, training | None |
| Collision | Serious injury | 4 | 1 | 4(L) | Marked pathways & Trolley wheels check | None |
| Material falling | Toe injury | 4 | 2 | 8 (M) | Proper racking & Storage | None |
| Sharp edges from material | Minor cuts | 2 | 2 | 4 (L) | PPES | None |
| Material Lift Fall | Physical Injury | 4 | 1 | 4 (L) | Lift maintenance |  |

2. Machining Process

| Hazard | Risk | S | L | R | Control Measures | Further Action |
| --- | --- | --- | --- | --- | --- | --- |
| Skin Contact with Cutting oil | Skin irritation | 2 | 5 | 10(M) | Use of Gloves | None |
| Slippage | Injury | 3 | 1 | 3 (L) | Cleaning | None |
| Electrical Shock | Fatal Injury | 5 | 1 | 5 (VH) | Earthing of Machines | SOP for Electrical Hazard |
| Fumes generation | Breathing Issue | 2 | 5 | 10 (M) | PPEs | None |
| Chips generation | Cut Injury | 3 | 3 | 9(M) | Gloves | None |
| High Air pressure during cleaning | Physical Injury | 3 | 3 | 9 (M) | Air Gun on each Air pipes | None |
| Fire due to Electrical Spark | Fatal Injury | 5 | 1 | 5 (VH) | Fire equipments Available | Fire Equipments checking & Mock drills |

3. Final Inspection

| Hazard | Risk | S | L | R | Control Measures | Further Action |
| --- | --- | --- | --- | --- | --- | --- |
| Contact with Oil | Skin Allergy | 2 | 5 | 10 (M) | PPEs & Regular Washing | None |
| Electrical Shock | Fatal Injury | 5 | 1 | 5 (VH) | Earthing of Machines | SOP for Electrical Hazard |
| Manual handling | Back strain | 3 | 3 | 9 (M) | Trolley use, training | None |

4.House Keeping & Cleaning Process

| Hazard | Risk | S | L | R | Control Measures | Further Action |
| --- | --- | --- | --- | --- | --- | --- |
| Skin Contact with Washing chemical | Skin Alergy | 1 | 5 | 10 (M) | Skin Washing with Soap | None |
| Sharp edges from material | Minor cuts | 2 | 2 | 4 (L) | PPES | None |
| Electrical Shock | Fatal Injury | 5 | 1 | 5 (VH) | Earthing of Machines | SOP for Electrical Hazard |

5. PACKING

| Hazard | Risk | S | L | R | Control Measures | Further Action |
| --- | --- | --- | --- | --- | --- | --- |
| Manual handling | Back strain | 2 | 4 | 8 (M) | Trolley use, training | None |
| Collision | Serious injury | 4 | 1 | 4 (L) | Marked pathways | None |
| Material falling | Head injury | 4 | 1 | 4 (L) | Proper racking | None |
| Sharp edges from material | Minor cuts | 2 | 2 | 4 (L) | PPES | None |

6. FINISHED GOODS STORAGE

| Hazard | Risk | S | L | R | Control Measures | Further Action |
| --- | --- | --- | --- | --- | --- | --- |
| Collapse due to Bins/Trolley Stacking at height | Fatal Injury | 5 | 1 | 5 (VH) | Currently no stacking on height | Max. Stack height display |
| Collision due to Material lifter movement | Physical Injury | 4 | 1 | 4 (L) | Traffic marking | None |

7. DISPATCH

| Hazard | Risk | S | L | R | Control Measures | Further Action |
| --- | --- | --- | --- | --- | --- | --- |
| Accident due to vehicle Movement | Fatal Injury | 5 | 1 | 5 (VH) | Security guard always be with Vehicle for assistance | Max Speed (10 km/hr) to be display |
| Collision due to Material lifter movement | Physical Injury | 4 | 1 | 4 (L) | Traffic marking | None |

8. Machine Maintenance

| Hazard | Risk | S | L | R | Control Measures | Further Action |
| --- | --- | --- | --- | --- | --- | --- |
| Tools fall on body | Physical Injury | 3 | 1 | 3 (L) | Trained Maintenance Personal | None |
| Slippage due to Oil spillage | Physical Injury | 4 | 1 | 4 (L) | Trained Maintenance Personal & immediate cleaning | None |
| Electrical Shock | Fatal Injury | 5 | 1 | 5 (VH) | Trained Maintenance Personal , Earthing of Machines | SOP for Electrical Hazard |

9. Pantry

| Hazard | Risk | S | L | R | Control Measures | Further Action |
| --- | --- | --- | --- | --- | --- | --- |
| Fire due to gas | Fatal Injury | 5 | 1 | 5 (VH) | Trained Pantry person & Gas cylinder taken from Authorised Agency only | Fire Extinguisher availability nearby |
| Slippage due to Water spillage | Physical Injury | 4 | 1 | 4 (L) | Trained Pantry person &i mmediate cleaning | None |
| Electrical Shock | Fatal Injury | 5 | 1 | 5 (VH) | Trained Maintenance Personal , Earthing of Machines | SOP for Electrical Hazard |